4-APRIL-2026

26

COAL THEORY

ASSIGNMENT NO.1

NAME: AYESHA EMAN                                                                    REG NO. 42536                                                                     PROGRAM: BS(CS)

## Part A:

### Q1. What is the distinction between computer organization and computer architecture?

### COMPUTER ARCHITECTURE:

Computer architecture refers to those attributes of a computer system that are visible to the programmer. These include the instruction set, data formats, addressing modes, and input/output mechanisms. It defines what the system does and how the system is designed from the programmer’s perspective.

COMPUTER ORGANIZATION:

Computer organization, on the other hand, refers to the internal operational units and their interconnections that realize the architectural specifications. It deals with hardware details such as control signals, interfaces, memory technology, and data paths. It explains how the system is implemented.

### Q2. What is the distinction between computer structure and computer function?

### COMPUTER STRUCTURE:

Computer structure refers to the way in which the components of a computer are interconnected. It describes the arrangement and relationship between hardware components such as the CPU, memory, and input/output units.

COMPUTER FUNCTION:

Computer function refers to the operation of each individual component as part of the overall system. For example, the CPU processes data, memory stores data and instructions, and input/output devices handle communication with external devices.

### Q3. What are the four main functions of a computer?

A computer performs four basic functions:

- Data Processing: The computer performs arithmetic and logical operations on data using the ALU.

- Data Storage: The computer stores data and instructions in memory for future use.

- Data Movement: The computer transfers data between internal components and external devices (input/output).

- Control: The control unit directs and coordinates all operations of the computer system.

### Q4. List and briefly define the main structural components of a computer.

The main structural components of a computer system are:

- Central Processing Unit (CPU): The core component that executes instructions and processes data.

- Main Memory: Stores programs and data currently in use by the CPU.

- Input/Output (I/O) Modules: Allow the computer to communicate with external devices such as keyboards, monitors, and printers.

- System Interconnection (Bus): A communication pathway that connects all components and transfers data, addresses, and control signals.

### Q5. List and briefly define the main structural components of a processor.

A processor consists of the following components:

- Arithmetic Logic Unit (ALU): Performs arithmetic operations (addition, subtraction, etc.) and logical operations (AND, OR, NOT).

- Control Unit (CU): Controls the operation of the processor by directing the flow of data between the CPU and other components.

- Registers: Small, high-speed storage locations within the CPU used to hold temporary data and instructions during execution.

### Q6. What is a stored program computer?

A stored program computer is a system in which program instructions and data are stored together in the same memory. The CPU retrieves instructions directly from memory and executes them sequentially. This concept allows programs to be easily modified and executed without changing the hardware.

### Q7. Explain Moore’s Law.

Moore’s Law states that the number of transistors on an integrated circuit doubles approximately every two years. As a result, computing power increases while the cost per transistor decreases. This trend has led to rapid advancements in computer performance and miniaturization of electronic devices.

### Q8. List and explain the key characteristics of a computer family.

A computer family refers to a group of computers that share a common architecture but differ in performance and cost. The key characteristics include:

- Common Instruction Set: All systems in the family support the same instructions.

- Compatibility: Programs written for one system can run on other systems in the same family.

- Scalability: Systems are available in different performance levels to meet various user needs.

- Similar Architecture: All systems follow the same basic design principles.

### Q9. What is the key distinguishing feature of a microprocessor?

The key distinguishing feature of a microprocessor is that the entire central processing unit (CPU) is integrated onto a single integrated circuit (chip). This makes microprocessors compact, cost-effective, and widely used in modern computing devices.

# Part B:

## Q1.  IAS Program:

### (a) Using Simplified Equation:

In the IAS machine, instructions are executed sequentially using memory operations. A typical program would involve:

- Loading values from memory into the accumulator

- Performing arithmetic operations

- Storing the result back in memory

Example:

LOAD M(X)

ADD M(Y)

STORE M(Z)

### (b) Hard Way:

In this method, each step of the computation is carried out separately, and intermediate results are stored in memory.

Example:

LOAD M(A)

MUL M(B)

STORE M(TEMP)

LOAD M(TEMP)

ADD M(C)

STORE M(RESULT)

This approach requires more instructions and memory usage.

## Q2.

### (a) Machine Code Instruction to Load Address 2:

In IAS, each instruction consists of an opcode and an address field.

Example instruction:

LOAD M(2)

Machine-level representation (conceptual):

0001 000000000010

Where:

- 0001 = opcode for LOAD

- Remaining bits = memory address

### (b) Number of Memory Trips:

The CPU must access memory twice:

- To fetch the instruction

- To fetch the data from the specified memory location

Therefore, the total number of memory accesses is two.

## Q3.  Describe Memory Read and Write Operations:

### Memory Read Operation:

- The CPU places the address of the required data into the Memory Address Register (MAR)

- A READ signal is sent through the control bus

- The memory retrieves the data and places it into the Memory Buffer Register (MBR)

- The data is transferred to the CPU through the data bus

### Memory Write Operation:

- The CPU places the target address into MAR

- The data to be written is placed into MBR

- A WRITE signal is sent through the control bus

- The data is stored in the specified memory location

### Role of System Buses:

- Address Bus – Carries memory addresses

- Data Bus – Transfers actual data

- Control Bus – Sends control signals such as READ and WRITE